Title: Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

Aim

To simulate and analyze the operating characteristics of microwave semiconductor devices and investigate the frequency-conversion characteristics of a microwave mixer using MATLAB.

Matlab Program:

clc;

clear;

close all;

%% Objective 1: Gunn Diode Characteristics

V = 0:0.1:10;

I = zeros(size(V));

for n = 1:length(V)

if V(n) <= 3

I(n) = 0.02 * V(n);

elseif V(n) <= 6

I(n) = 0.06 - 0.01 * (V(n)-3);

else

I(n) = 0.03 + 0.015 * (V(n)-6);

end

end

%% Objective 2: Schottky Diode I-V Characteristics

Vd = -0.5:0.01:0.5;

Is = 1e-9;

eta = 1.2;

Vt = 0.026;

Id = Is * (exp(Vd/(eta*Vt)) - 1);

Id = max(min(Id, 0.1), -0.1);

%% Objective 3: Mixer Frequency Conversion

fRF = 10e6;

fLO = 8e6;

fIF = abs(fRF - fLO);

fSum = fRF + fLO;

t = 0:1e-9:2e-6;

RF = cos(2*pi*fRF*t);

LO = cos(2*pi*fLO*t);

MixerOutput = RF .* LO;

%% Objective 4: Frequency Spectrum Analysis

N = length(MixerOutput);

Fs = 1/(t(2)-t(1));

Y = abs(fft(MixerOutput));

f = (0:N-1)*(Fs/N);

halfN = floor(N/2);

fMHz = f(1:halfN)/1e6;

Ym = Y(1:halfN);

Ym = Ym/max(Ym);

%% 3x3 Dashboard

figure('Name','Microwave Semiconductor and Mixer Analysis',...

'NumberTitle','off');

subplot(3,3,1);

plot(V,I,'b','LineWidth',1.5);

grid on;

xlabel('Voltage (V)');

ylabel('Current (A)');

title('Gunn Diode I-V');

subplot(3,3,2);

plot(V,I,'r','LineWidth',1.5);

grid on;

xlabel('Voltage (V)');

ylabel('Current (A)');

title('Gunn Diode NDR Region');

subplot(3,3,3);

plot(Vd,Id,'k','LineWidth',1.5);

grid on;

xlabel('Voltage (V)');

ylabel('Current (A)');

title('Schottky Diode I-V');

subplot(3,3,4);

plot(t*1e6,RF,'b');

grid on;

xlabel('Time (\mus)');

ylabel('Amplitude');

title('RF Signal');

subplot(3,3,5);

plot(t*1e6,LO,'r');

grid on;

xlabel('Time (\mus)');

ylabel('Amplitude');

title('Local Oscillator Signal');

subplot(3,3,6);

plot(t*1e6,MixerOutput,'m');

grid on;

xlabel('Time (\mus)');

ylabel('Amplitude');

title('Mixer Output Signal');

subplot(3,3,7);

bar([fIF fRF fLO fSum]/1e6,...

[1 0.5 0.5 1]);

grid on;

set(gca,'XTickLabel',{'IF','RF','LO','Sum'});

ylabel('Relative Level');

title('Frequency Components');

subplot(3,3,8);

plot(fMHz,Ym,'LineWidth',1.5);

grid on;

xlim([0 25]);

xlabel('Frequency (MHz)');

ylabel('Normalized Magnitude');

title('Mixer Output Spectrum');

subplot(3,3,9);

axis off;

text(0.05,0.85,'Microwave Mixer Results',...

'FontSize',12,'FontWeight','bold');

text(0.05,0.65,sprintf('RF Frequency = %.1f MHz',fRF/1e6));

text(0.05,0.48,sprintf('LO Frequency = %.1f MHz',fLO/1e6));

text(0.05,0.31,sprintf('IF Frequency = %.1f MHz',fIF/1e6));

text(0.05,0.14,sprintf('Sum Frequency = %.1f MHz',fSum/1e6));

sgtitle('Simulation-Based Investigation of Microwave Semiconductor Devices and Mixer');

sgtitle('Simulation-Based Investigation of Microwave Semiconductor Devices and Mixer');Expected Output:

| Parameter | Expected output |
| --- | --- |
| Gunn diode peak current | 0.8A |
| Reference Frequency | 5GHz |
| RF input frequency | 10MHz |
| Local oscillator frequency | 8MHz |
| Intermediate frequency | 2MHz |
| Sum frequency | 18MHz |
| Conversion loss | 6dB |

Output:

MICROWAVE MIXER SIMULATION RESULTS

---------------------------------

RF Frequency          = 10.00 MHz

LO Frequency          = 8.00 MHz

Intermediate Frequency= 2.00 MHz

Sum Frequency         = 18.00 MHz

Conversion Loss       = 6.00 dB

Plots: