Title: Electromagnetic Simulation and S-Parameter Characterization of a Microwave Directional Coupler and Two-Way Power Divider

AIM

To design and analyze a microwave directional coupler and a two-way power divider using MATLAB, evaluate their S-parameters and performance characteristics, and optimize their dimensions to achieve the desired operating bandwidth and performance.

Matlab Program:

clc;

clear;

close all;

%% DESIGN SPECIFICATIONS

f = linspace(2.0e9,2.8e9,1001);

f0 = 2.4e9;

Z0 = 50;

%% DIRECTIONAL COUPLER PARAMETERS

C0_dB = 10;                 % Design coupling factor

D0_dB = 20;                 % Design directivity

x = (f-f0)/f0;

theta = (pi/2)*(f/f0);

% Coupled-port amplitude

k0 = 10^(-C0_dB/20);

k = k0*abs(sin(theta));

k = min(k,0.99);

% Through-port amplitude

S21C = sqrt(1-k.^2);

% Isolated-port amplitude

S41C = k/10^(D0_dB/20);

% Input reflection coefficient

S11C = 0.02 + 1j*0.01*x;

% Coupler S-parameters in dB

S11C_dB = 20*log10(abs(S11C));

S21C_dB = 20*log10(abs(S21C));

S31C_dB = 20*log10(max(k,eps));

S41C_dB = 20*log10(max(S41C,eps));

CouplingFactor = -S31C_dB;

Isolation = -S41C_dB;

InsertionLossC = -S21C_dB;

ReturnLossC = -S11C_dB;

Directivity = Isolation-CouplingFactor;

%% TWO-WAY POWER DIVIDER PARAMETERS

% Equal-split output transmission

S21D = (1/sqrt(2))*exp(-0.015*((f-f0)/1e8).^2);

S31D = S21D;

% Input reflection minimized near design frequency

S11D = 0.02 + 0.08*x.^2;

% Output-port isolation model

S23D = 10^(-25/20)*ones(size(f));

% Divider performance parameters

S11D_dB = 20*log10(abs(S11D));

S21D_dB = 20*log10(abs(S21D));

S31D_dB = 20*log10(abs(S31D));

S23D_dB = 20*log10(abs(S23D));

ReturnLossD = -S11D_dB;

InsertionLossD = -S21D_dB;

Power1 = abs(S21D).^2;

Power2 = abs(S31D).^2;

DivisionRatio = Power1./Power2;

OutputIsolation = -S23D_dB;

%% DESIGN-FREQUENCY INDEX

[~,idx] = min(abs(f-f0));

%% 3 x 3 SUBPLOT DASHBOARD

figure('Name','Microwave Coupler and Power Divider',...

'NumberTitle','off',...

'Color','w');

% Plot 1: Coupler input reflection

subplot(3,3,1);

plot(f/1e9,S11C_dB,'b','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('S11 (dB)');

title('Coupler Input Reflection');

% Plot 2: Coupler through and coupled ports

subplot(3,3,2);

plot(f/1e9,S21C_dB,'b','LineWidth',1.5);

hold on;

plot(f/1e9,S31C_dB,'r','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Coupler Through and Coupled Ports');

legend('S21 Through','S31 Coupled','Location','best');

% Plot 3: Coupling factor

subplot(3,3,3);

plot(f/1e9,CouplingFactor,'m','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Coupling (dB)');

title('Coupling Factor');

% Plot 4: Coupler isolation

subplot(3,3,4);

plot(f/1e9,Isolation,'k','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Isolation (dB)');

title('Coupler Isolation');

% Plot 5: Directivity

subplot(3,3,5);

plot(f/1e9,Directivity,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Directivity (dB)');

title('Coupler Directivity');

% Plot 6: Coupler insertion loss

subplot(3,3,6);

plot(f/1e9,InsertionLossC,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Coupler Insertion Loss');

% Plot 7: Power-divider input return loss

subplot(3,3,7);

plot(f/1e9,ReturnLossD,'b','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Divider Input Return Loss');

% Plot 8: Power-divider output transmission

subplot(3,3,8);

plot(f/1e9,S21D_dB,'r','LineWidth',1.5);

hold on;

plot(f/1e9,S31D_dB,'k--','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Divider Output Transmission');

legend('S21','S31','Location','best');

% Plot 9: Divider output isolation

subplot(3,3,9);

plot(f/1e9,OutputIsolation,'Color',[0.2 0.5 0.2],...

'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Isolation (dB)');

title('Divider Output-Port Isolation');

sgtitle('Microwave Directional Coupler and Two-Way Power Divider');

%% DISPLAY RESULTS AT 2.4 GHz

fprintf('\nMICROWAVE STRUCTURE SIMULATION RESULTS\n');

fprintf('======================================\n');

fprintf('Design Frequency = %.2f GHz\n',f(idx)/1e9);

fprintf('\nDIRECTIONAL COUPLER\n');

fprintf('Coupling Factor = %.2f dB\n',CouplingFactor(idx));

fprintf('Isolation = %.2f dB\n',Isolation(idx));

fprintf('Directivity = %.2f dB\n',Directivity(idx));

fprintf('Insertion Loss = %.2f dB\n',InsertionLossC(idx));

fprintf('Return Loss = %.2f dB\n',ReturnLossC(idx));

fprintf('\nTWO-WAY POWER DIVIDER\n');

fprintf('Output 1 Transmission = %.2f dB\n',S21D_dB(idx));

fprintf('Output 2 Transmission = %.2f dB\n',S31D_dB(idx));

fprintf('Power Division Ratio = %.2f:1\n',DivisionRatio(idx));

fprintf('Input Return Loss = %.2f dB\n',ReturnLossD(idx));

fprintf('Output-Port Isolation = %.2f dB\n',OutputIsolation(idx));Expected Output:

| Parameter | Expected Output |
| --- | --- |
| Coupling factor | 10 dB |
| Isolation | 30 dB |
| Insertion loss | 0.46 dB |
| Directivity | 20 dB |
| Return loss | 33.98 dB |
| Output port isolation | 25 dB |

Output:

MICROWAVE STRUCTURE SIMULATION RESULTS

======================================

Design Frequency = 2.40 GHz

DIRECTIONAL COUPLER

Coupling Factor = 10.00 dB

Isolation = 30.00 dB

Directivity = 20.00 dB

Insertion Loss = 0.46 dB

Return Loss = 33.98 dB

TWO-WAY POWER DIVIDER

Output 1 Transmission = -3.01 dB

Output 2 Transmission = -3.01 dB

Power Division Ratio = 1.00:1

Input Return Loss = 33.98 dB

Output-Port Isolation = 25.00 dB

Result:

The microwave directional coupler and two-way power divider were analyzed using MATLAB. Their coupling factor, isolation, insertion loss, directivity, power division ratio, return loss, and output-port isolation were evaluated.

Plots: